550Watts

MONOCRYSTALLINE SOLAR MODULE

Products features:

- Half Cut Bifacial Solar panel

- Strong aluminum alloy frame

- Standard tolerance (Guaranteed standard tolerance from -5~+5% ensures power output reliability)

- Excellent weak light performance

Excellent performance under low light environments (mornings, evenings, and cloudy days)

- Withstand high wind and snow loads

Entire module certified to withstand high wind loads (2400 Pascal) and snow loads (5400 Pascal)

Power parameters

| Model No. | SFM-550W |
| --- | --- |
| Maximum Power at STC (Pmax) | 550W |
| Optimum Operating Voltage (Vmp) | 41.95V |
| Open - Circuit Voltage (Voc) | 49.8V |
| Optimum Operating Current (Imp) | 13.12A |
| Short - Circuit Current (Isc) | 13.98A |
| Operating Module Temperature | -40 °C / +85 °C |
| Maximum System Voltage | 1500V |
| Diodes | 0 to 100% |
| Power Tolerance | 3 by pass |
| Maximum Power at STC (Pmax) | -3/+3 % |

STC: irradiance 1000 W/m2, module temperature 25 °C, AM=1.5; Power measurement tolerance: ± 5%

Mechanical Characteristics

| Solar Cell | Monocrystalline 182*91mm |
| --- | --- |
| No. of Cells | 144(6× 24) |
| Dimensions | 2279*1134*35*35 mm |
| Weight | 27.3Kgs |
| Front Glass | 3.2 mm (0.13 inches) tempered glass |
| Frame | Anodized aluminum alloy |
| Junction Box | IP68 rated |
| Output Cables | TUV |
|  | 4.0 mm2 (0.16inches2), symmetrical lengths (-) 22 mm 8.66 inches) and (+) 22mm (8.66inches) |
| Connectors | RADOX® SOLAR integrated twist locking connectors |

Temperature Characteristics

| Nominal Operating Cell Temperature (NOCT) | 45±2 ℃ |
| --- | --- |
| Temperature Coefficient of Pmax | -0.35%/℃ |
| Temperature Coefficient of Voc | -0.276%/℃ |
| Temperature Coefficient of Isc | 0.046%/℃ |

uality and Safety:

Product warranty:

Singfo Solar modules will last for at least 25 years.

Free from defects in material and workmanship for 5 years.

All the panels passed IEC, TUV standards.

Power warranty:

Maintain more than 90% of minimum rated power for 10 years.

Maintain more than 80% of minimum rated power for 25 years.